SAFETY

# Aerial lift pre-use inspection checklist

The check before a boom lift or scissor lift is used each day: a walk-around with the key off, then a function test of every control, including the emergency controls.

How often: Daily, before use   ·   Who: Lift operator   ·   About 10 min   ·   17 items

| Site or area |  | Date and time |  |
| --- | --- | --- | --- |
| Asset or line |  | Completed by |  |

Before you use it

- OSHA 29 CFR 1926.453(b)(2) requires the controls of extensible and articulating boom platforms to be tested each day before use, and allows only authorized persons to operate an aerial lift.

- The manufacturer’s operating manual sets the full pre-start inspection for each make and model. Add its items here and remove those that do not apply.

## Walk-around, key off

| NO. | ITEM | RESULT | NOTES |
| --- | --- | --- | --- |
| 1 | Scan the lift’s tag | Visited |  |
| 2 | Manual on board; decals and capacity plate legible | Pass Fail N/A |  |
| 3 | Tires and wheels undamaged | Pass Fail N/A |  |
| 4 | No hydraulic leaks; hoses and cylinders sound | Pass Fail N/A |  |
| 5 | Battery, fuel and hydraulic fluid levels correct | Pass Fail N/A |  |
| 6 | Guardrails, gate and anchor points secure | Pass Fail N/A |  |
| 7 | Outriggers or stabilizers work, where fitted | Pass Fail N/A |  |

## Function test

| NO. | ITEM | RESULT | NOTES |
| --- | --- | --- | --- |
| 8 | Ground controls work, including the emergency stop | Pass Fail N/A |  |
| 9 | Platform controls work, including the emergency stop | Pass Fail N/A |  |
| 10 | Emergency lowering works | Pass Fail N/A |  |
| 11 | Tilt alarm and motion alarms work | Pass Fail N/A |  |
| 12 | Drive, brakes and steering work | Pass Fail N/A |  |

## Work area

| NO. | ITEM | RESULT | NOTES |
| --- | --- | --- | --- |
| 13 | Surface firm and level; no holes or drop-offs | Pass Fail N/A |  |
| 14 | No overhead power lines or obstructions in reach | Pass Fail N/A |  |

## Result

| NO. | ITEM | RESULT | NOTES |
| --- | --- | --- | --- |
| 15 | Result | Safe to use Tagged out of service |  |
| 16 | Photo of any defect | Photo taken |  |
| 17 | Operator | Name ______________________________ Signature ______________________________ Date ______________________________ |  |

## Follow-up

For each failed check or reading out of limit: what was found, what was done, and the work order raised for it.

| NO. | WHAT WAS FOUND | ACTION OR WORK ORDER | OWNER | DUE |
| --- | --- | --- | --- | --- |

## References

- OSHA 29 CFR 1926.453, aerial lifts

Free to use and adapt. To run it on mobile, with checkpoint scans, readings checked against their limits as they are entered, and photos and signatures attached to the check, see vardian.io/templates/aerial-lift-pre-use-inspection.