SAFETY

# Alloy steel chain sling inspection checklist

The daily check of each chain sling before use and the thorough periodic inspection OSHA requires at least every 12 months, with the record of the month each sling was last inspected.

How often: Each day before use; thorough inspection at least every 12 months   ·   Who: Competent person designated by the employer   ·   About 5 min   ·   19 items

| Site or area |  | Date and time |  |
| --- | --- | --- | --- |
| Asset or line |  | Completed by |  |

Before you use it

- OSHA 29 CFR 1910.184(d) requires each sling and all fastenings and attachments to be inspected for damage or defects each day before use by a competent person designated by the employer, with additional inspections during use where service conditions warrant. Damaged or defective slings are removed from service immediately.

- For alloy steel chain slings, 1910.184(e)(3) adds a thorough periodic inspection on a regular basis set by the frequency of use, the severity of service conditions, the nature of the lifts and experience with similar slings, and in no event at intervals greater than once every 12 months. The employer makes and maintains a record of the most recent month in which each sling was thoroughly inspected and makes it available for examination. The thorough inspection covers wear, defective welds, deformation and increase in length.

- A sling comes out of service when the chain size at any point of a link is less than the minimum in Table N-184-1, when a hook is cracked, opened more than 15 percent of its normal throat opening or twisted more than 10 degrees from the plane of the unbent hook, or when a master link, coupling link or other component is cracked or deformed (1910.184(e)(8) and (e)(9)). A sling heated above 1000 °F is removed permanently (1910.184(e)(6)).

- Each sling carries permanently affixed durable identification stating size, grade, rated capacity and reach (1910.184(e)(1)); makeshift links or fasteners formed from bolts or rods are not used (1910.184(e)(2)); and the proof-test certificate for each new, repaired or reconditioned sling is retained (1910.184(e)(4)). Add the manufacturer’s inspection criteria for your grade of chain.

## Sling

| NO. | ITEM | RESULT | NOTES |
| --- | --- | --- | --- |
| 1 | Scan the sling tag | Visited |  |
| 2 | Inspection type | Daily check before use Thorough periodic inspection |  |
| 3 | Identification tag permanently affixed and legible: size, grade, rated capacity and reach | Pass Fail N/A |  |
| 4 | Sling identification as marked |  |  |
| 5 | Month of the last thorough inspection on record |  |  |

## Chain

| NO. | ITEM | RESULT | NOTES |
| --- | --- | --- | --- |
| 6 | Links free of nicks, gouges, cracks and defective welds | Pass Fail N/A |  |
| 7 | No stretched or elongated links; reach matches the identification | Pass Fail N/A |  |
| 8 | No bent, twisted or kinked links | Pass Fail N/A |  |
| 9 | Chain size at the most worn point of a link Remove from service if less than the minimum allowable size in Table N-184-1 for the chain size. | ____________ inches Limit ____________ |  |
| 10 | No heat damage, discoloration or weld spatter | Pass Fail N/A |  |
| 11 | No excessive corrosion or pitting | Pass Fail N/A |  |

## Hooks and attachments

| NO. | ITEM | RESULT | NOTES |
| --- | --- | --- | --- |
| 12 | Hooks not cracked, throat opening not spread more than 15 percent, not twisted more than 10 degrees | Pass Fail N/A |  |
| 13 | Hook latches, where fitted, close and hold | Pass Fail N/A |  |
| 14 | Master links, coupling links and rings not cracked, deformed or worn | Pass Fail N/A |  |
| 15 | Attachments rated at least equal to the chain; no makeshift links, bolts or rods | Pass Fail N/A |  |

## Result

| NO. | ITEM | RESULT | NOTES |
| --- | --- | --- | --- |
| 16 | Result | Fit for use Removed from service |  |
| 17 | Photo of any defect | Photo taken |  |
| 18 | Record updated with the month of this thorough inspection | Pass Fail N/A |  |
| 19 | Competent person | Name ______________________________ Signature ______________________________ Date ______________________________ |  |

## Follow-up

For each failed check or reading out of limit: what was found, what was done, and the work order raised for it.

| NO. | WHAT WAS FOUND | ACTION OR WORK ORDER | OWNER | DUE |
| --- | --- | --- | --- | --- |

## References

- OSHA 29 CFR 1910.184, slings

Free to use and adapt. To run it on mobile, with checkpoint scans, readings checked against their limits as they are entered, and photos and signatures attached to the check, see vardian.io/templates/chain-sling-inspection-checklist.