SAFETY

# Walking-working surfaces and housekeeping inspection checklist

The regular inspection OSHA 1910.22(d) requires of floors, aisles, stairs, platforms and passageways: clean, dry and orderly surfaces, hazards guarded until they are repaired, loads and safe access.

How often: Weekly, and as necessary   ·   Who: Area supervisor   ·   About 15 min   ·   18 items

| Site or area |  | Date and time |  |
| --- | --- | --- | --- |
| Asset or line |  | Completed by |  |

Before you use it

- OSHA 29 CFR 1910.22(d)(1) requires walking-working surfaces to be inspected “regularly and as necessary” and maintained in a safe condition. It sets no interval, so the weekly frequency here is a starting point for the site to adjust.

- Surface conditions under 1910.22(a): all places of employment, passageways, storerooms, service rooms and walking-working surfaces kept clean, orderly and sanitary; each workroom floor clean and, to the extent feasible, dry, with drainage maintained and dry standing places such as false floors, platforms and mats provided where wet processes are used; and surfaces maintained free of hazards such as sharp or protruding objects, loose boards, corrosion, leaks, spills, snow and ice.

- A hazardous condition must be corrected or repaired before an employee uses the surface again. If that cannot be done immediately, the hazard must be guarded to prevent employees from using the surface until it is corrected or repaired (1910.22(d)(2)). A correction or repair that involves the structural integrity of the surface must be performed or supervised by a qualified person (1910.22(d)(3)).

- Each walking-working surface must support its maximum intended load (1910.22(b)), and the employer must provide, and ensure each employee uses, a safe means of access to and from walking-working surfaces (1910.22(c)). Add the site’s own items for guardrails, floor holes and dock edges, which other sections of subpart D cover.

- Run the checklist per area or building section, and attach a photo to every hazard so the work order shows the exact spot.

## Area

| NO. | ITEM | RESULT | NOTES |
| --- | --- | --- | --- |
| 1 | Scan the area tag | Visited |  |
| 2 | Area or building section |  |  |
| 3 | Surface type | Floor and aisles Stairs and ramps Platform or mezzanine Dock or loading area Outdoor walkway |  |

## Surface condition (1910.22(a))

| NO. | ITEM | RESULT | NOTES |
| --- | --- | --- | --- |
| 4 | Floors, passageways and storerooms clean, orderly and free of debris | Pass Fail N/A |  |
| 5 | Floor dry; spills and leaks cleaned up, and drainage working where wet processes run | Pass Fail N/A |  |
| 6 | Dry standing places such as mats, false floors or platforms in place at wet processes | Pass Fail N/A |  |
| 7 | No sharp or protruding objects, loose boards or corrosion | Pass Fail N/A |  |
| 8 | No snow or ice on walkways and outdoor surfaces | Pass Fail N/A |  |
| 9 | Aisles and passageways clear for their full width | Pass Fail N/A |  |

## Loads and access (1910.22(b)–(c))

| NO. | ITEM | RESULT | NOTES |
| --- | --- | --- | --- |
| 10 | Stored materials within the rated load of the floor, platform or mezzanine | Pass Fail N/A |  |
| 11 | Safe means of access and egress to each platform, mezzanine and elevated surface in place and in use | Pass Fail N/A |  |
| 12 | Stairs, ramps and ladders used for access in sound condition | Pass Fail N/A |  |

## Hazards found (1910.22(d))

| NO. | ITEM | RESULT | NOTES |
| --- | --- | --- | --- |
| 13 | Each hazard corrected before the surface is used again, or guarded off until it is | Pass Fail N/A |  |
| 14 | Repairs affecting structural integrity assigned to, or supervised by, a qualified person | Pass Fail N/A |  |
| 15 | Photo of each hazard | Photo taken |  |
| 16 | Location and description of each hazard |  |  |

## Result

| NO. | ITEM | RESULT | NOTES |
| --- | --- | --- | --- |
| 17 | Result | No hazards Hazards guarded, work orders raised Area closed until repaired |  |
| 18 | Inspector | Name ______________________________ Signature ______________________________ Date ______________________________ |  |

## Follow-up

For each failed check or reading out of limit: what was found, what was done, and the work order raised for it.

| NO. | WHAT WAS FOUND | ACTION OR WORK ORDER | OWNER | DUE |
| --- | --- | --- | --- | --- |

## References

- OSHA 29 CFR 1910.22, walking-working surfaces: general requirements

Free to use and adapt. To run it on mobile, with checkpoint scans, readings checked against their limits as they are entered, and photos and signatures attached to the check, see vardian.io/templates/walking-working-surfaces-inspection.